LoRa

Attached to USART3

| LoRa TX | USART3_RX (PB11) |
| --- | --- |
| LoRa RX | USART3_TX (PB10) |
| LoRA M0 | PC5 |
| LoRA M1 | PB0 |
| LoRa AUX | PB12 |

Baud 9600

USART3 and USART4 IRQ are common (shared)

Interrupt is enabled in CubeMX

LoRa Configuration Code

A 6-byte packet defined at the start of the file lorae32_stm32.c

The LoRa E32 module has in built pullups on M0 and M1 pins, so by default they are in SLEEP mode. We must also make the associated GPIO pins in HIGH default state in CubeMX.